Proposed Solution:

Bio-inspired USV that efficiently collects surface waste from water bodies. The USV features biomimetic fin-based propulsion for agility and energy efficiency, allowing it to navigate smoothly through water with minimal disturbance. It incorporates a dual navigation system: manual control via a radio transmitter for primary navigation and autonomous mode using Pixhawk flight controller and Raspberrypi for more complex, pre-programmed waste collection routes. The primary mission is to aid in environmental clean-up efforts by collecting floating waste on water surfaces, such as plastic debris, leaves, and other pollutants.

Conceptual Design Approach:

3D Model / Design:

a) Isometric View

b) Top View

c) Side View

d) Front View

e) Back View

2D Draft:

Technical Requirements and Specifications:

Functional Requirements:

- Primary Navigation via Radio Transmitter: Allows for direct, manual control by the operator to navigate around large or concentrated waste areas.

- Autonomous Navigation using Pixhawk: Capable of self-guided routes to follow predetermined paths along water surfaces for routine waste collection.

- Waste Collection Mechanism: Equipped with a front-mounted One-Way Mechanical Barrier collection system that captures and stores floating debris.

- Bio-Inspired Undulating Fin from knife fish: Designed for energy-efficient, low-disturbance movement, particularly beneficial in sensitive or ecologically significant areas.

- Obstacle Avoidance: Equipped with Camera to detect and avoid obstacles such as rocks, buoys, or other vessels.

Navigation and Control System:

- In manual mode Pixhawk receives input from the radio transmitter, enabling direct control.

- In autonomous mode, Pixhawk manages movement based on GPS waypoints and real-time IMU adjustments, providing stability and autonomous navigation for waste collection.

- Waste Collection System:

- One-Way Mechanical Barrier: Positioned at the front of the USV, this system includes a rake system that collects up floating debris as the USV moves through the water. The collected waste is stored in a detachable bin or basket located at the back of the USV for easy removal and disposal.

- Mesh Filter: A mesh filter integrated into the collection system ensures that only debris and waste are collected, allowing water to flow through and reducing drag on the vehicle.

- Propulsion System: Biomimetic Fin Design:

Inspired by the efficient movement of marine animals, the fin-based propulsion system enhances maneuverability and minimizes disturbances in the water. The fins:

- Operate with oscillating movement, mimicking natural swimming, which reduces energy usage and maintains a quiet presence, avoiding excessive disruption to aquatic life.

- Provide versatile maneuverability, especially beneficial for navigating around obstacles or reaching confined spaces where waste tends to accumulate.

- Pixhawk 2.4.8 for Autonomous Stability and Control:

- IMU (Inertial Measurement Unit): Provides stability control by tracking roll, pitch, and yaw, essential for steady movement and accurate waste collection.

- GPS Module: Enables precise navigation for autonomous operations, allowing the USV to follow defined paths or return to base when necessary.

- 1080p HD Camera:

- Integrated camera captures surface images for obstacle detection and visual navigation using YOLO v7 algorithms.

- Raspberry Pi as Companion Computer:

- The Raspberry Pi serves as the companion computer, handling image processing (YOLO v7), sensor data integration, and supporting autonomous navigation alongside the Pixhawk controller.

- Waste Bin Full Detection:

- An IR sensor detects when the waste bin is full, triggering the USV to return to the home position for unloading.

- Home Position for Waste Unloading:

- The USV autonomously returns to a pre-programmed home position using Pixhawk and GPS waypoints for waste unloading.

Hull Design and Structural Integrity

The hull is designed to ensure:

- Stability and Buoyancy: Lightweight, corrosion-resistant materials are used for the hull to prevent rusting and degradation over time.

- Waterproof Compartment for Electronics: The Pixhawk, raspberry pi 4, GPS module and other control electronics are secured in a waterproof compartment using acrylic pipe enclosed by resin, ensuring safety and durability in aquatic environments.

Software stacks:

- Autodesk Fusion 360: Used for designing the USV components (hull, waste collection system, propulsion), including part modeling and assemblies.

- ANSYS 2024 R1: Analyzes structural integrity and hydrodynamics, simulating stress, strain, and fluid flow to ensure durability and efficiency.

- QGroundControl: Provides mission planning, waypoint navigation, and real-time telemetry for precise monitoring and control of the USV.

- ROS 2: Manages sensor integration (GPS, IMU, ultrasonic), processing data for autonomous navigation and obstacle avoidance.

- Gazebo: Simulates USV dynamics, navigation, and waste collection in 3D for design validation and optimization before deployment.

- YOLO v7: Processes visual input from the camera for real-time obstacle detection and navigation assistance.

Electronic circuit Design:

Theoretical Calculations:

1                                                                                2

3         					  4

Power Calculation:

Preliminary Analysis and Simulations:

Hydrodynamic and Structural Analysis:

- Drag Coefficient Testing: Computational Fluid Dynamics (CFD) simulations confirmed that the hull and fin designs minimize water resistance, optimizing speed and efficiency for waste collection.

- Finite Element Analysis (FEA): Ensured the hull’s durability under potential forces from wave impact and collected debris.

  - Velocity Flow

  - Static Pressure Flow

  - Drag Force

Source: Ansys report

Project Tentative Budget:

- Production Cost: ₹171,207 per unit.

- Operational Expenses: ₹85,000 - ₹100,000/month.

- Revenue: Sales, subscriptions, data analytics, recycling partnerships.

- Expansion: Pilot phase, regional expansion, nationwide growth.

- Funding: ₹50-70 lakh.

Project Timeline:

Team Expertise: